FVE4K-KIT Configuration Tool user instruction

Catalogue

1.	Summary	1

1.1	Brief introduction	1

1.2	Installation instructions	1

1.3	The device connection methods are divided into two types: RS-232 port and network	2

1.4	Some suggestions and explanations	6

2.	Status bar description and Question Mark Button Description	7

2.1	The status bar is mainly divided into four parts, as shown in the following figure.	7

2.2	Question Mark Button Description	8

3.	Setting Page Description	8

3.1	Switch RX and TX configuration pages	8

3.2	RX Configuration Page Description	9

3.3	TX Configuration Page Description	10

4.	Upgrade Page Description	11

4.1	RX Upgrade Page Description	11

4.2	TX Upgrade Page Description	14

5.	Frequently Asked Questions	18

5.1	Common issues with network port mode	18

5.2	Common issues with serial port mode	20

Summary

Brief introduction

This document is used to introduce the use of Blustream FVE4K-KIT Configuration Tool running on 64 bit windows PC.

Version 1.5.1 or above Configuration Tool can work with NXP MCU firmware version 1.0.q or above.

The Configuration Tool screenshot images in this document could be showing multiple version numbers, please ignore this and take them as 1.5.1.

Installation instructions

- Installation environment

This software is applicable to win7 64 bit, win10 64 bit, win11 64 bit Bitwise operation operating systems 32 Bitwise operation is not supported.

- Double click on the installation software to start the installation.

- Select the option 'Anyone who uses this computer (all users)' and click 'Next' to proceed to the next step.

- Select the installation directory and click Install to install.

- Wait for the installation to complete, click on the Finish button to complete the installation.

The device connection methods are divided into two types: RS-232 port and network

- Connect using a network connection

When TX and RX are connected together using fiber optics, using a computer RJ-45 port directly or a computer RJ-45 port through network switch to connect the 1Gb LAN port of TX or RX can control both TX and RX.

If the fiber optic is not connected, only devices connected to 1Gb LAN ports can be controlled.

Except for the RJ-45 connected to the device, all virtual network cards or WIFI in the computer must be disabled as shown in the following figure.

Modify the computer network configuration so that the computer and device are on the same network segment as shown in the following figure.

Note: The default IP address of the device is 169.254.xxx.xxx, and the subnet mask is 255.255.0.0.

Select “IP” from the Interface Port and click Connect As shown in the following figure:

Prompt for device search results after successful connection.

- Connect using serial port

When TX and RX are connected together using optical fiber, both TX and RX can be controlled by connecting the RS-232 ports of TX or RX using a computer.

If the fiber optic is not connected, only devices connected to the RS-232 port can be controlled.

Select “RS-232” for the interface port, and select the port number and Baud for the COM port to connect to the device, and click Connect as shown in the following figure.

Note: The baud-rate selection must match the baud-rate setting of the device RS-232 port. If you forget the baud-rate setting of the device RS-232 port, it can be read out via “IP” mode or factory reset it to 57600 which is default.

After successful connection, the following prompt will be displayed.

Some suggestions and explanations

- When upgrading batch devices, it is recommended to first upgrade one end of the device TX or RX), and then upgrade the other end of the device RX or TX.

Taking upgrade on batch TX as an example, the upgrade process:

- Connect the 1G network port of the TX device to the PC, connect the power supply, and power on.

- Wait for TX device boot up completely (link light and video light flashing ends)

- Select interface port, select “IP” mode, and click Connect.

- Switch to the Update page and select the TX upgrade page.

- Select MCU or AVP firmware and click on the MCU or AVP Upgrade button.

- Waiting for upgrade to complete.

- Replace the next TX device.

- Connect the 1G network port of the TX device to the PC, connect the power supply, and power on.

- Wait for the new TX device boot up completely (link light and video light flashing ends).

- Click on the MCU or AVP Upgrade button directly to upgrade.

- Waiting for upgrade to complete.

- Repeat (7), (8), (9), (10), (11) until all TX upgrades on batch TX are completed.

- When configuring device parameters (such as EDID, IR, etc.) for batch devices, it is recommended to use serial port mode for configuration.

In the serial port mode, the bottom status bar displays TX: Found, and it can be directly controlled by replacing another TX device (the Baud of the serial port is the same). The bottom status bar displays RX: Found. It can be directly controlled by replacing another RX device (the Baud of the serial port is the same).

In IP mode to configure device parameters, because the MAC address of the device is required to control the device, after replacing the device, it is necessary to click the Connect button again to search for connections in order to control the device.

Status bar description and Question Mark Button Description

The status bar is mainly divided into four parts, as shown in the following figure.

1: Connection status.

2: Device Search Status.

3: Instruction operation results, or the time taking when processing Reboot instruction, Reset instruction, Firmware Upgrade operations.

4: System time.

Question Mark Button Description

Click on the top right corner button “?”, the button will open the Logs folder, which stores the operation logs of the tool.

Setting Page Description

Switch RX and TX configuration pages

- Click on RX to switch to the RX configuration page

- Click on TX to switch to the TX configuration page

RX Configuration Page Description

Link: ON/OFF, 10G fiber link status.

Video: YES/NO, video on transmission or not.

Link LED: Control Link LED with four configurations: DEFAULT, ON, OFF, FLASH.

DEFAULT: Default state, no manual external control.

ON: LED light always on

OFF: LED light always off

FLASH: LED light 1HZ flashing

Video LED: Control Video LED with four configurations: DEFAULT, ON, OFF, FLASH.

DEFAULT: Default state, no manual external control

ON: LED light always on

OFF: LED light always off

FLASH: LED light 1HZ flashing

HDMI Out: A switch that controls HDMI output, with two configurations: ON and OFF

ON: Turn on HDMI output

OFF: Turn off HDMI output

BaudRate: Configure serial port Baud of the device RS-232 port. There are eight configurations: 115200, 57600, 38400, 19200, 14400, 9600, 4800, 2400.

IR: Configure IR types, with two configurations: 5V and 12V

5V: Configure to use 5V IR cable

12V: Configure to use 12V IR cable

Reboot: Reboot the RX device.

Reset: RX device restores default settings. Note: Device name will not be reset.

TX Configuration Page Description

Link: ON/OFF, 10G fiber link status.

Video: YES/NO, video on transmission or not.

Link LED: Control Link LED with four configurations: DEFAULT, ON, OFF, FLASH.

DEFAULT: Default state, no manual external control.

ON: LED light always on

OFF: LED light always off

FLASH: LED light 1HZ flashing

Video LED: Control Video LED with four configurations: DEFAULT, ON, OFF, FLASH.

DEFAULT: Default state, no manual external control

ON: LED light always on

OFF: LED light always off

FLASH: LED light 1HZ flashing

EDID: Configure EDID.

BaudRate: Configure serial port Baud of the device RS-232 port. There are eight configurations: 115200, 57600, 38400, 19200, 14400, 9600, 4800, 2400.

IR: Configure IR types, with two configurations: 5V and 12V

5V: Configure to use 5V IR cable

12V: Configure to use 12V IR cable

Reboot: Reboot the TX device.

Reset: TX device restores default settings. Note: Device name will not be reset.

Upgrade Page Description

RX Upgrade Page Description

- Configure device name

Enter the device name in the input box and click Apply to submit.

Note: Device name is only available to change in “IP” mode.

- Upgrade AVP interface instructions

AVP Version: Current version

AVPMac: AVP MAC address

Select File: Select Upgrade File

Upgrade: Upgrade button

- AVP upgrade process

Click on Select File to select the upgrade file

Double click on the selected file and click on Upgrade

Wait for the upgrade to complete. The prompt 'Update AVP successfully' pops up in the upper right corner. The bottom status bar displays the upgrade time and prompts Update AVP successfully

After successful upgrade, you can replace other RX device and directly click the Upgrade button on AVP to upgrade to the next RX AVP.

Note: AVP Firmware Update is only available in “IP” mode.

- MCU Upgrade Interface Description

MCU Version: Current version

MCUMac: MCU MAC address

Select File: Select Upgrade File

Upgrade: Upgrade button

- MCU upgrade process

Click on Select File to select the upgrade file

Double click on the selected file and click on Upgrade

Wait for the upgrade to complete. The prompt 'Update MCU successfully' pops up in the upper right corner. The status bar displays the upgrade time and Update MCU success

After successful upgrade, you can replace other RX device and directly click the Upgrade button on the MCU to upgrade to the next RX MCU.

Note: MCU Firmware Update is only available in “IP” mode.

TX Upgrade Page Description

- Configure device name

Enter the device name in the input box and click Apply to submit.

Note: Device name is only available to change in “IP” mode.

- Upgrade AVP interface instructions

AVP Version: Current version

AVPMac: AVP MAC address

Select File: Select Upgrade File

Upgrade: Upgrade button

- AVP upgrade process

Click on Select File to select the upgrade file

Double click on the selected file and click on Upgrade

Wait for the upgrade to complete. The prompt 'Update AVP successfully' pops up in the upper right corner. The bottom status bar displays the upgrade time and prompts Update AVP successfully.

After the upgrade is successful, you can replace other TX device and directly click the Upgrade button on the AVP to upgrade to the next TX AVP.

Note: AVP Firmware Update is only available in “IP” mode.

- MCU Upgrade Interface Description

MCU Version: Current version

MCUMac: MCU MAC address

Select File: Select Upgrade File

Upgrade: Upgrade button

- MCU upgrade process

Click on Select File to select the upgrade file

Double click on the selected file and click on Upgrade

Wait for the upgrade to complete. The prompt 'Update MCU successfully' pops up in the upper right corner. The status bar displays the upgrade time and Update MCU success.

After the upgrade is successful, you can replace other TX device and directly click the Upgrade button on the MCU to upgrade to the next TX MCU.

Note: MCU Firmware Update is only available in “IP” mode.

Frequently Asked Questions

Common issues with network port mode

- The following prompt appears

1: Check whether the 1G network port of the device is connected to the computer RJ45 normally.

2: Check if the computer's settings are IP address: 169.254.xxx.xxx, subnet mask: 255.255.0.0.

3: Is the device booted up completely (link light and video flashing complete).

- Connection failure

1: Check whether controlserver.exe exists in Task manager

- Upgrade MCU failed

1: Check whether the network is normal.

2: Check if all virtual network cards and wifi are disabled except for the computer RJ45 connected to the device.

Common issues with serial port mode

- Connection failure

1: Check if the serial port number is correct.

2: Check if the serial port is occupied by other programs.

3: Confirm whether the serial port Baud is correct.

- TX&RX have been connected through fiber cable. Connecting to one end of the RS-232 port, clicking the Connect button can only find the local end.

1: Use BlueRiver tool to check if UART3 serial port pairing is correct.

2: Click to connect using the “IP” mode of the upper computer. After searching for both TX and RX, click Disconnect. Then switch to “RS-232” mode and click Connect to check if both TX and RX can be found.

3: Factory reset TX and RX devices.

2023-06-16