DA22DIG Upgrade Information

- Open the General Upgrade Tool

 UB

- Select the Com port for the USB to Serial cable (example below = Com5)

- Select 57600 Baudrate

- Click ’13 Byte’ so this is ticked

- Select the Connect button on the right to establish connection to the DA22DIG

When connected the MCU firmware number will be shown at the bottom of the program

- Once connected click the Open button.

- Select the file then press Open to accept it.

- Press Upgrade to start the process

- The firmware upgrade process will start and take approx. 10 seconds

You will receive a message to say when it is completed.