Contents:

Introduction:________________________________ Page 2 . Connecting to the Blustream ____________________Page 2.     Guest Information:____________________________Page 3 .           IR or RS-232 Guest Mode via Web GUI____________Page 3.                          IR or RS-232 Guest Mode via Telnet______________Page 6.                                         IR or RS-232 Guest Mode via Guest Mode_________Page 8.  Notes______________________________________Page 10.

Thank you for purchasing this

For optimum performance and safety, please read these instructions carefully before connecting, operating or adjusting this product. Please keep this manual for future reference.

Surge Protection Device

This product contains sensitive electrical components that may be damaged by electrical spikes, surges, electric shock, lightning strikes, etc. Use of surge protection systems is highly recommended in order to protect and extend the life of your equipment.

# Introduction.

This guide will walk through the Guest Mode functionality of the system. Guest mode is the ability to have a single connection from a 3rd party control system to the Matrix via TCP/IP or RS-232 and use this to route IR and RS-232 commands out to specific HDBaseT outputs of the Matrix, using the system as a method for signal extension, eliminating the need for additional control cables or connections from 3rd Party control systems.

# Connecting To the Blustream Matrix.

There are several methods of connecting to the matrix depending on your chosen form of communication:

Web GUI:

The matrix features a built in web GUI that allows configuration and control of the device. You can access this by entering the IP Address of the matrix in your web browser. By default the matrix is set to DHCP, however if a DHCP server (eg: network router) is not installed the matrix IP address will revert to below details. If there is no DHCP server in your network, or you are connected point to point directly to the Matrix, ensure that the IP address of your PC is within the same IP range as the default IP address below.

IP Address is: 192.168.0.200                  Default Username is: blustream             Default Password is: 1234

Telnet:

The matrix allows API control via Telnet. You can access the telnet port using your favourite telnet software with the below address details:

Default IP Address is: 192.168.0.200                  Default Telnet Port: 23

RS-232:

The RS-232 port is used for configuration and control of the Matrix, as well as pass through of RS-232 commands to a

compatible HDBaseT receiver. You can access the RS-232 port using your favourite serial software. Note that your device will need to be connected to the matrix via an appropriate serial adapter.

The default RS-232 communication settings are:

Baud rate: 57600  		Data bit: 8   		Stop bit: 1  		Parity bit: none

.








Guest Mode Information.

Guest mode is designed to allow control of 3rd party devices via the matrix via an RS-232 connection or by sending IR commands to control a device local to the HDBT receiver in each zone. There are 3 ways for Guest Mode to function within the matrix as follows:

• IR or RS-232 Guest Mode via Web GUI

• IR or RS-232 Guest Mode via Telnet Port 23 or RS-232

• IR or RS-232 Guest Mode via Telnet Guest Mode Port

IR or RS-232 Guest Mode via Web GUI

IR or RS-232 Guest Mode via the Web GUI allows you to send IR commands or RS-232 control strings directly from the Web GUI of the matrix. The following steps outline this process, and assume your matrix and receiver are correctly wired and functioning both to each other, and the 3rd party device you wish to control:

- Connect to the Web GUI of the matrix, login to the admin section, and navigate to the Guest Mode tab.

- Select the Status “On/Off” button associated with the input / output (certain models have input Guest mode) that you wish to use for Guest Mode on, and toggle it to the on position. You will receive the following pop up which will advise you that all Telnet connections will be disconnected from the matrix and you will need to reconnect them. Press YES to proceed or No to Cancel.

- Guest mode provides several settings per output as follows:

• Telnet Port - This is the Telnet port you will need to connect to in order to send commands and receive feedback from the remote device connected via RS-232

• Status On/Off - This enables or disabled the guest mode port for this output

• Baud Rate - Sets the communication baudrate for the guest mode port

• Commands ASCII/HEX - Sets the command format for RS-232 between ASCII text or HEX characters

• Direction One-direction/Bi-direction - Sets whether the RS-232 guest mode is configured in One-direction mode where it sends commands and ignores any feedback, or in Bi-direction mode where it sends commands and waits for the delay period to receive feedback

• Delay Time (s) - Sets the delay time in which the matrix waits for feedback from the RS-232 guest mode port after sending a Bi-directional command

• RS-232 & IR Test Command - This is where you enter your RS-232 string or IR HEX command you wish to transmit. The matrix automatically identifies if it is an RS-232 string or an IR command based on the command structure. Please note that you can enter multiple lines in this box. It may be necessary to use a blank line as a carriage return as a termination character for some 3rd party devices

• Send Button - Press this button to send out the command via the corresponding output port of the matrix

- To send an RS-232 command simply enter your command in the RS-232 & IR Command box and press the Send button. If the command is sent successfully, it will return a success pop up message. In Bi-direction mode, the matrix will wait for feedback from the connected device before it provides a pop up message. If the delay period times out and the matrix does not receive feedback, it will pop up with an error message advising it did not receive feedback. If you receive a failed message from a Rs-232 command please add a new line and try to send the command again. Check that your command string is correct and verify that the 3rd party device is connected correctly and try and send the command again. In One-direction mode, the matrix will send the command and provide a success message regardless of whether the connected device functions correctly or not.

- To send an IR command simply enter the IR command in Pronto HEX format into the RS-232 & IR Command box and press the Send button. If the command is sent successfully, it will return a success pop up message. If there is an issue with the IR command, the pop up will provide an error message.

An example IR HEX command is available below which is a simple Power Toggle command for a Samsung TV:

0000 006d 0022 0003 00a9 00a8 0015 003f 0015 003f 0015 003f 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 003f 0015 003f 0015 003f 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 003f 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0015 0040 0015 0015 0015 003f 0015 003f 0015 003f 0015 003f 0015 003f 0015 003f 0015 0702 00a9 00a8 0015 0015 0015 0e6e

- Check your 3rd party device to confirm that the command you have sent works and functions as intended.

IR or RS-232 Guest Mode via Telnet

IR or RS-232 Guest Mode via Telnet allows you to send IR commands or RS-232 control strings via API commands from the Telnet port of the matrix. You can use any Telnet software to carry out this task such as HyperTerm, Hercules, Putty, SSCOM or Uart Assist. For this manual we will be using SSCOM.

The API commands relevant for guest mode are as follows:

OUT xx GUEST ON/OFF    Set System Guest Mode xx On Or Off

xx = 00: All HDBT output

xx = 01: HDBT output 1

(Please note, this will disconnect any active telnet connections to this matrix)

OUT xx IRHEXSEND b     Send Pronto Hex Code To Remote End

xx = 00: All HDBT output

xx = 01: HDBT output 1

b  = IR Value(Hex Format)

OUT xx RS232SEND yy:z:a:b   Send b To RS232 Port xx And Wait yy Seconds For Feedback

xx = 00: Local RS232 Port

xx = 01: HDBT output 1 RS232 Port

yy = [00-10]: Wait For The Feedback dd Seconds

z  = a ASCII, h HEX

a  = 1 2400, 2 4800, 3 9600, 4 19200, 5 38400, 6 57600 (Default), 7 115200

b  = RS232 command

- Open your Telnet software and connect to the IP address of the matrix and port 23. Press Connect to start sending commands.

- Enter the API command you wish to transmit and press send.
For example, to send an IR HEX command from output 1 I would send the following command:

OUT 01 IRHEXSEND 0000 0067 000D 0000 0060 0018 0030 0018 0030 0018 0030 0018 0030 0018 0018 0018 0030 0018 0018 0018 0030 0018 0018 0018 0018 0018 0018 0018 0018 03DA

If successful, you should receive a [SUCCESS] message back as follows:

[SUCCESS]OUT 01 IRHEXSEND 0000 0067 000D 0000 0060 0018 0030 0018 0030 0018 0030 0018 0030 0018 0018 0018 0030 0018 0018 0018 0030 0018 0018 0018 0018 0018 0018 0018 0018 03DA .:

- 3. An example RS-232 command is available below which is a simple Power Toggle command for an AV Receiver:

OUT 01 RS232SEND 1:A:6:pon

If successful, you should receive a [SUCCESS] message back as follows, and the 3rd party device should power on:

- The same control method can also be carried out via an RS-232 connection from your PC/Control System to the matrix. Instead of using the IP address and telnet port, simply select your Serial COM port and send commands in the exact same way as the previous steps:

- Ensure that you select the correct baud rate for the matrix - 57600 is the default setting. Then press OpenCom to connect and to start sending commands.

IR or RS-232 Guest Mode via Guest Mode Port:

IR or RS-232 Guest Mode via Guest Mode Port allows Bi-directional RS-232 control strings to be transmitted and received, and IR commands to be sent to a 3rd party device. You can use any Telnet software to carry out this task such as HyperTerm, Hercules, Putty, SSCOM or Uart Assist. For this manual I will be using SSCOM. No additional API commands are required.

 Open your Telnet software and connect to the IP address of the matrix. The port number for each output can be found in the Guest Mode Tab in the web GUI and follows the format 500x where x is the output of the matrix. For example, output 1 would use port 5001, output 2 would use port 5002, etc..

- Open your Telnet software and connect to the IP address of the matrix. The port number for each output can be found in the Guest Mode Tab in the web GUI and follows the format 500x where x is the output of the matrix. For example, output 1 would use port 5001, output 2 would use port 5002, etc..

- 2. Once connected via Guest Mode Port, you can send and receive commands to/from the 3rd party device connected at the other end as if you were connected directly to the product.

For IR commands, simply send the entire IR HEX command. No prefix or control API is necessary.

For RS-232 commands, simply send the 3rd party device’s control commands. No additional prefix or control API is required

- For example, the below screenshot shows a connection to Guest Mode Port 5001, and feedback from the STATUS command of a device connected via RS-232 from the HDBT receiver of output 1:

NOTES: